# IoT smart irrigation and monitoring system

A sensor-driven irrigation and environment monitoring system built around an Arduino and a Raspberry Pi. Soil moisture, temperature, and light are read continuously and used to control a solenoid valve, a heating/cooling relay, and a lighting relay. A Raspberry Pi exposes live readings over a web dashboard.

Arduino C++ Raspberry Pi Apache/PHP MySQL MQTT (planned) Embedded systems

## Status

This repo covers two layers, and they are at different stages:

- Core system - soil moisture, temperature, light, and water level sensing, with pump/solenoid/relay control. This was physically built and tested as a bachelor's project.

- Extended design - humidity, rain, pH, and flow sensing, dual-channel temperature control, protection components (flyback diodes, dividers, decoupling), and an MQTT/cloud connectivity layer. This is designed and documented (schematic-level and in firmware) but not yet physically built or tested. It's flagged as such throughout this README and in docs/.

## Features

- Soil-moisture-triggered irrigation via a solenoid valve

- Rain and humidity used as overrides, so irrigation doesn't run needlessly (extended design)

- Temperature-based heating and cooling on two independent relay channels

- Light-based LED/relay control

- Water tank level detection via a 4-probe ladder, with automatic refill

- Water usage tracking via a flow sensor (extended design)

- Local web dashboard on the Raspberry Pi; MQTT/cloud publishing planned (extended design)

## System architecture

The system is organized into four layers: sensing, control, actuation, and connectivity. See docs/architecture.png for the full block diagram and docs/circuit-diagram.png for wiring.

Sensors -> Microcontroller (decision logic) -> Actuators

|

Raspberry Pi -> Dashboard / MQTT / Cloud

## Hardware

| Component | Purpose | Status |
| --- | --- | --- |
| Soil moisture sensor | Triggers irrigation | Built & tested |
| NTC temperature sensor | Drives heating/cooling | Built & tested |
| LDR (light sensor) | Drives lighting relay | Built & tested |
| Water level probes (4-stage) | Tank refill logic | Built & tested |
| Solenoid valve | Field irrigation | Built & tested |
| Relay modules | Switch loads | Built & tested |
| Water pump + motor driver | Fills tank | Built & tested |
| Raspberry Pi 3 | Local dashboard | Built & tested |
| DHT22 (humidity) | Irrigation override | Designed, not built |
| Rain sensor | Irrigation override | Designed, not built |
| Soil pH sensor | Nutrient monitoring | Designed, not built |
| Water flow sensor | Usage tracking | Designed, not built |
| Flyback diodes | Protects transistors from inductive spikes | Designed correction |
| Voltage divider resistors | Required for LDR and NTC readings | Designed correction |
| Decoupling capacitor | Protects MCU from switching noise | Designed correction |

Full pin-level wiring is in docs/connection-map.md.

## How it works

See docs/working-mechanism.md for the full technical explainer, including the irrigation override logic, the NTC-to-Celsius conversion, and the water level and flow subsystems.

## Repository structure

iot-smart-irrigation-system/

├── README.md

├── firmware/

│   └── irrigation_controller.ino

├── raspberrypi/

│   ├── dashboard/

│   └── setup.md

├── docs/

│   ├── working-mechanism.md

│   ├── connection-map.md

│   ├── architecture.png

│   └── circuit-diagram.png

├── results/

│   └── test-log.md

└── LICENSE

## Setup

- Flash firmware/irrigation_controller.ino to the Arduino (Arduino IDE or arduino-cli).

- Wire components per docs/connection-map.md.

- On the Raspberry Pi, install Apache and PHP, then deploy raspberrypi/dashboard/ (see raspberrypi/setup.md).

- Connect Arduino to the Pi via USB for serial data.

- Power on and confirm sensor readings appear on the dashboard.

## Results

From the original built-and-tested core system: the tank refill sequence was verified across all four probe stages, with the pump activating on empty and shutting off at full. Soil-moisture-triggered irrigation and light/temperature-driven relay switching were verified to respond correctly to sensor thresholds during testing. See results/test-log.md for details.

The extended sensors and connectivity layer have not been tested, since they exist only at the design/firmware level.

## Known limitations

- Extended sensors (humidity, rain, pH, flow) are unverified — pH in particular requires physical calibration against buffer solutions before its readings mean anything.

- Local dashboard only; MQTT/cloud publishing is designed but not wired up in the current dashboard code.

- No data logging over time in the original design — readings are live-only, not historical.

## License

MIT