# Working mechanism

This document explains how each subsystem makes its decisions. For pin-level wiring, see connection-map.md. For the full firmware, see ../firmware/irrigation_controller.ino.

## Overview

The system reads soil moisture, temperature, light, water level, humidity, rain, soil pH, and water flow. Each sensor feeds a threshold-based (or, for water level and flow, count-based) decision that drives one actuator. The irrigation path checks rain and humidity as overrides before acting. A separate section below covers the small support and protection components (resistors, capacitor, diodes) that don't sense or actuate anything but the system doesn't work correctly without.

## Sequential flow: sensing -> control -> actuation -> connectivity

The sections below explain each subsystem individually, but it's worth stating the pipeline explicitly, since that's the actual architecture:

- Sensing layer. All eight sensors (soil moisture, temperature, light, water level, humidity, rain, pH, flow) take a physical quantity — resistance, capacitance, a voltage, a pulse train — and turn it into a value the microcontroller can read on an analog or digital pin. Nothing here makes a decision; this layer only measures.

- Control layer. The microcontroller reads every sensor value once per loop cycle, applies the threshold or override logic described below (dry soil, but check rain and humidity first; too hot or too cold, on separate channels; count wet probes), and decides what each actuator should do. The voltage regulator and decoupling capacitor here don't participate in that decision-making — they keep the control layer electrically stable while it happens (see Support and protection components).

- Actuation layer. The microcontroller's decisions are carried out physically: the solenoid opens or stays shut, the pump runs or stops, the heating or cooling relay energizes, the light relay switches. Each of these is a higher-current or higher-voltage load than the microcontroller can drive directly, which is why relays, a motor driver, and a switching transistor sit between the control layer and the actual hardware — and why the flyback diodes exist at this boundary specifically.

- Connectivity layer. Independent of the control loop above, the microcontroller also writes one line of sensor readings to serial each cycle. The Raspberry Pi picks that line up, serves it on a local dashboard, and — in the designed extension — would publish it to an MQTT broker for remote/cloud access. This layer only reports on what the other three layers are doing; it has no influence over the irrigation, temperature, or lighting decisions themselves.

Layers 1-3 form a closed loop that repeats every cycle. Layer 4 is a one-way tap off that loop — it observes, it doesn't act back into the system.

## Soil moisture -> irrigation

Sensing principle: the probe pair acts as a variable resistor. Dry soil is a poor conductor, so resistance is high and the sensor's analog output is low. Wet soil conducts well, so resistance drops and the output rises.

Signal path: analog pin A0, range 0-1023.

Control logic: the soil reading alone is not enough to open the solenoid. Three conditions are checked in order:

- Is the soil dry (above SOIL_DRY_THRESHOLD)?

- If dry — is rain currently detected? If yes, skip irrigation; the rain will do the job.

- If no rain — is ambient humidity above HUMIDITY_HIGH_PCT? If yes, skip irrigation; high humidity plus already-adequate moisture risks fungal disease.

Only if soil is dry, there's no rain, and humidity isn't already high does the solenoid open. This override chain is designed content — the original build only checked soil moisture directly.

Actuator response: solenoid valve opens (relay energized) or stays closed.

## Temperature -> heating/cooling

Sensing principle: the NTC thermistor's resistance falls as temperature rises (negative temperature coefficient). Reading it through a voltage divider produces a value the ADC can interpret; converting that to an actual Celsius value requires the beta-equation approximation, not a straight-line formula, since the thermistor's resistance-temperature curve is nonlinear.

Signal path: analog pin A1, converted in readTemperatureC().

Control logic: two independent comparisons — tempC > TEMP_HIGH_C (too hot) and tempC < TEMP_LOW_C (too cold). These can never both be true, since they compare against different thresholds.

Actuator response: two separate relay channels, PIN_COOLING and PIN_HEATING. The original design used one shared relay pin for both loads, which cannot actually distinguish "too hot" from "too cold" — this was corrected in the firmware.

## Humidity (DHT22) -> irrigation override input (designed extension)

What it measures: ambient relative humidity, plus temperature as a secondary reading (unused here since the NTC already covers temperature).

Sensing principle: the DHT22 contains a capacitive humidity sensing element — a moisture-absorbing dielectric between two electrodes, whose capacitance changes with humidity. Unlike the analog sensors elsewhere in this system, the DHT22 does the analog-to-digital conversion internally and sends a finished reading out over a single data line using a timing-based protocol (bit values are encoded as pulse durations, not voltage levels).

Signal path: digital pin D11, read via the DHT library, which handles the timing protocol.

Why it's here: soil moisture alone can't tell you whether watering is a good idea right now. High ambient humidity combined with already-wet soil raises fungal disease risk rather than helping the crop — this sensor gives the irrigation logic (see above) the second input it needs to make that call.

## Rain detection -> irrigation override input (designed extension)

What it measures: whether rain is currently falling.

Sensing principle: works on the same principle as the soil moisture sensor — a resistive plate exposed to open air. Water bridging the plate's traces lowers resistance, raising the analog output; a dry plate reads low.

Signal path: analog pin A3.

Why it's here: without it, the irrigation logic would open the solenoid on a dry-soil reading even while rain was actively falling on the same field — wasted water and an obviously wrong decision a farmer would never make manually. This sensor lets the override chain treat rain as a hard stop, checked before humidity.

Sensing principle: the LDR's resistance falls as light increases. Read through a voltage divider (a bare LDR produces no usable voltage on its own).

Signal path: analog pin A2.

Control logic: single comparison against LIGHT_LOW_THRESHOLD.

Actuator response: light relay on when below threshold, off otherwise.

## Water level -> tank refill pump

Unlike the sensors above, this isn't a threshold check — it's a count. Four probes are placed at increasing heights in the tank. Water is conductive, so each submerged probe closes its own circuit, read as a digital HIGH. The number of HIGH probes indicates how full the tank is (0 = empty, 4 = full — the original build used a 9-probe ladder for finer resolution; this firmware simplifies to 4 stages, documented in code).

Actuator response: pump runs while stage is below the "full" count, stops once all probes are wet.

## Soil pH -> nutrient monitoring (designed extension)

What it measures: soil acidity/alkalinity, which governs how well crops can actually absorb the nutrients already in the soil — even correctly watered, fertilized soil underperforms outside the right pH range for a given crop.

Sensing principle: an ion-selective glass electrode generates a small voltage proportional to hydrogen ion concentration in the surrounding soil solution. That voltage is amplified by the probe module into a range the ADC can read.

Signal path: analog pin A4, converted in readPH().

Why calibration is non-negotiable here: every other analog sensor in this system reads a physical quantity fairly directly — soil moisture and light both map roughly linearly from raw ADC value to a usable threshold. pH doesn't. The relationship between the electrode's output voltage and actual pH is sensor-specific and drifts over time, so the probe must be calibrated against known pH 4 and pH 7 buffer solutions before its readings mean anything. The slope/offset values in the firmware are placeholders for exactly this reason — they're not a substitute for doing that calibration.

## Water flow -> usage tracking (designed extension)

The flow sensor outputs a pulse per unit volume passed. Because pulses arrive faster than the main loop runs, they're counted via a hardware interrupt (flowISR) rather than a regular read. Once per second, the accumulated pulse count is divided by the sensor's calibration constant (FLOW_PULSES_PER_LITRE) and added to a running total. This is the only subsystem that produces a cumulative value rather than a live decision.

## Support and protection components

These parts don't sense or actuate anything themselves, but the system doesn't work correctly — or survive — without them. Grouped here since they're easy to treat as an afterthought, which is exactly how they got left out of the original report.

LDR and NTC voltage divider resistors (10k each) What: a fixed resistor in series with the LDR/thermistor, forming a voltage divider. How: neither device outputs a voltage on its own — they only change resistance. Placing a fixed resistor in series and reading the midpoint voltage is what converts "changing resistance" into "a voltage the ADC can read." Why 10k: chosen to be close to the sensor's resistance at typical operating conditions, which keeps the divider sensitive in the range that actually matters.

Transistor base resistor (1k) What: a resistor between the LDR's divider output and the switching transistor's base. How: limits current into the base — without it, the transistor draws excessive current and either burns out or fails to switch cleanly. Why: any time a low-power signal (like the LDR reading) switches a higher-power load (the relay) through a transistor, the base needs current limiting.

Flyback diode (1N4007), across the relay coil and the solenoid coil What: a diode placed in reverse bias directly across each inductive load. How: while the coil is energized, the diode is reverse-biased and does nothing. When power is cut, the collapsing magnetic field induces a voltage spike (back-EMF) that can be many times the supply voltage; the diode gives that spike a safe path to dissipate through the coil instead of arcing through the switching transistor. Why it matters: this was missing from the original report entirely. Any inductive load — a relay coil, a solenoid, a motor — needs this. Skipping it is how switching transistors get destroyed during testing, often without an obvious cause.

LED current-limiting resistors (220-330 ohm) What: a resistor in series with each indicator LED. How: LEDs have very low internal resistance, so connected directly across a 5V supply they draw far more current than they're rated for and burn out almost immediately. The resistor limits current to a safe level. Why 220-330 ohm: a reasonable range for a standard LED at 5V; exact value depends on the LED's forward voltage and desired brightness.

DHT22 pull-up resistor (10k) What: a resistor between the DHT22's data line and 5V. How: the DHT22's one-wire protocol needs the line to sit at a defined HIGH state between transmissions; without a pull-up, the line floats and reads are unreliable. Why: standard requirement for this class of digital sensor, called out in the datasheet.

Decoupling capacitor (0.1uF ceramic), near the microcontroller's power pins What: a small capacitor across Vcc and GND, placed physically close to the MCU. How: absorbs high-frequency electrical noise generated when nearby relays switch, preventing that noise from causing the MCU to reset or misread a sensor at the moment a relay clicks. Why: standard practice on any board switching inductive loads near sensitive digital logic — cheap insurance against intermittent, hard-to-diagnose faults.

Voltage regulator (5V output), between the 12V supply and the low-voltage circuitry What: a component that takes the higher supply voltage used by relays and the pump (12V) and steps it down to a stable 5V for the microcontroller and sensors. How: it continuously compares its output against a fixed reference and adjusts to hold that output steady even as the input fluctuates, dissipating the difference as heat (which is why it's typically paired with a small heat sink). Why: without it, the 12V rail that powers the pump/relays could find its way into circuitry built for 5V — through a wiring mistake, a shared ground fault, or simply running the wrong wire to the wrong pin — and destroy the microcontroller or sensors. It's the boundary that keeps the high-current and low-current halves of the circuit from taking each other down.

Serial line protection (Arduino <-> Raspberry Pi) What: either a USB connection (preferred) or a voltage divider on the Arduino-to-Pi line if wiring directly to GPIO. How: Arduino outputs 5V logic; Raspberry Pi GPIO pins are 3.3V-only and can be damaged by a direct 5V input. USB serial sidesteps the issue entirely since it goes through the Arduino's onboard USB-to-serial chip, not raw GPIO. Why it's called out: this is a real way to damage a Raspberry Pi, not a theoretical concern — worth deciding explicitly rather than wiring GPIO-to-GPIO by default.

The Arduino sends one CSV line of readings over USB serial each cycle. The Raspberry Pi reads this line and serves it on a local dashboard via Apache/PHP. Publishing to an MQTT broker for remote/cloud access is designed but not implemented in the current dashboard code — see the README's "Known limitations."